# Errata

- Recommendation: Use a AA battery

- USB C needs resistors on CC1 and CC2 to negotiate power. Solution: 5.1k each from CC1 and CC2 to ground

- Resistors for each LED need to be increased from 470 to something like 1k

  - Voltage across Green LED resistor at VCC = 5 🡪 2.4V 🡪5.1mA

  - Blue LED 🡪 2.2V 🡪 4.7mA

| Vcc | 5 | 2.8 |
| --- | --- | --- |
| Resistor VRed |  |  |
| Resistor Vgreen | 2.4 | 0.45 |
| Resistor Vblue | 2.2 | 0.24 |
| Total IRed (mA) | 272 | 86 |
| Total IGreen (mA) | 265 | 53 |
| Total IBlue (mA) | 222 | 29 |

5V output draws too much power from the coin cell battery, so supply voltage is dropping and VCC cannot be maintained. Fix: Lower Vcc. This can be done by changing R76 to the following:

- 680k: ~5v

- 360k: 2.9V

- 345k: 2.8V

- 330k: 2.7V

- 71k: ~2.77V

## Summary of changes to keep battery life reasonable

- R76 🡪 680k, buck71utput it 2.68V instead of 5V

- R99 is 47k instead of 470 (Green)

- R98 is 33k instead of 470 (Red)

- R100 is still 470 ohm (blue LEDs have higher Vf)

# Second Revision

- Add a higher ntroller and fill in the rest of the LEDs to make a grid

  - STM32L05x promising

- 24x8

- AAA battery x 2 for combined, ~1AH each for 2AH. 40mA continuous draw = 50 hours

- RGB option

- Possible PWM options

  - PCA9635

  - PCA9685

  - tlc5940 (too expensive)

# Font

https://imgur.com/a/bitpotion-free-4x7-pixel-font-KjeUx

# Development plan

LED array is 19x7

38 blank spots

-->95 LEDs per board

If using 3mm TH LEDs, 4mm per led,

- LED: C22461786 (2.7x2.8mm)

- Boost converter:

  - TPS63002 (C116655),

  - inductor (C9890)

- Flip flop: C6762 / SN74HC74DR

- BJT: C2150 / 300mW.Vce sat = 500mv, 🡪 600mA max

- USB C connector: C165948

- Battery holder: C964806

- 10uf cap: C1713 (0805 X5R 16V)

- 100nF cap: C1591 (0603 50v X7R)

Unit cost > 100:

Lithium ion batteries: 1.48Wh

AA batteries: 2-4Wh

CR2450: 0.59540

Boost: 0.1235

$500 manual assembly fee

- Either LEDs, switch, battery, or USBC

- Not USBC

- Not switch

- Not battery

- It is the batteries

https://www.snapeda.com/parts/TYPE-C-31-M-12/HRO%20Electronics%20Co.,%20Ltd./view-part/

Senior banquet is may 2nd

Order right now, 4/7 in china, PCB gets built on 4/10. PCB is assembled by 4/16, arrives 4/20

Cost of prototype run: base time is 3 days PCB, 5-6 days assembly 🡪 $145 pre shipping.

3-4 days assembly adds $95 for a total of $240

1 week turnaround, i.e. shipping on 4/14, arrives 4/21. Order on 4/21

Full batch run with expedited assembly will cost $500 pre shipping,

Order right now, first PCB is assembled on 4/13. If that PCB works, do a batch

Options:

Get JLC to test a board for us:

- Order 10 prototypes right now, boards done on 4/14. Gets delivered by 22nd-24th

- Order 50 on the 14th, boards done on 26th, ship on 26th, arrive

- Double checked LED footprint

- Double checked stop mask

- Double checked BJT pinout

- Double checked switch

# Battery life measurement

- Turned on at 3:19PM with 324k resistor (Vcc = 2.66)

- Lasts about 40 minutes